EX.NO:12								        DATE: 03.04.2025

OPTICAL FLOW USING LUCAS KANADE AND FEATURE TRACKING USING KANADE LUCAS TOMASI ALGORITHM

Aim:

To estimate Optical Flow of first two Frames of the video and the full video and

Feature Tracking using Kanade Lucas Tomasi Algorithm.

Algorithm:

Step 1: Video Setup

- Load the video using cv2.VideoCapture().

- Read the first frame to prepare for manual object selection.

Step 2: Region Selection (ROI)

- Manually select the bounding box by right-clicking to define corners.

- Display the rectangle live on the frame.

- Middle-click to reset coordinates.

- After pressing ESC, crop the selected area.

Step 3: Feature Detection

- Convert the cropped ROI to grayscale.

- Use cv2.goodFeaturesToTrack() to detect strong corner features inside this ROI.

- Filter the feature points to ensure they lie within the selected box.

- Store the first valid point (for optical flow tracking).

Step 4: Optical Flow Initialization

- Define Lucas-Kanade parameters, including window size, pyramid levels, and termination criteria.

- Read the first frame again and convert to grayscale.

- Initialize a mask to store drawing trails and begin frame counter and timer.

Step 5: Optical Flow Tracking Loop

- Loop through all frames:

  - Convert each frame to grayscale.

  - If p0 (tracking point) exists:

    - Use cv2.calcOpticalFlowPyrLK() to calculate the new position of the tracked point.

    - If tracking was successful:

      - Extract good_new and good_old points.

      - Draw line and circle showing movement.

      - Update p0 and previous frame.

      - Display FPS on screen.

    - Else:

      - Set p0 = None (tracking lost).

- Check if the tracked point goes out of bounds; if yes, reset p0.

Step 6: Feature Re-Detection (If Tracking Lost)

- If p0 is None (no valid tracking point):

- Redetect new features in the current frame using cv2.goodFeaturesToTrack().

- Reset drawing mask to clean state.

- Continue tracking newly found point.

Step 7: Exit Condition

- Press ESC to break the loop.

- Release video capture and close all OpenCV windows.

Code:

import numpy as np
import cv2
import matplotlib.pyplot as plt

def optical_flow(old_frame, new_frame, window_size=5, min_quality=0.01):
    max_corners = 1000
    min_distance = 1
    feature_list = cv2.goodFeaturesToTrack(old_frame, max_corners, min_quality, min_distance)

    w = window_size // 2

    old_frame = old_frame / 255.0
    new_frame = new_frame / 255.0

    kernel_x = np.array([[-1, 1], [-1, 1]])
    kernel_y = np.array([[-1, -1], [1, 1]])
    kernel_t = np.array([[1, 1], [1, 1]])

    fx = cv2.filter2D(old_frame, -1, kernel_x)
    fy = cv2.filter2D(old_frame, -1, kernel_y)
    ft = cv2.filter2D(new_frame, -1, kernel_t) - cv2.filter2D(old_frame, -1, kernel_t)

    u = np.zeros(old_frame.shape)
    v = np.zeros(old_frame.shape)

    for feature in feature_list:
        j, i = feature.ravel()
        i, j = int(i), int(j)

        if i - w < 0 or i + w + 1 > old_frame.shape[0] or j - w < 0 or j + w + 1 > old_frame.shape[1]:
            continue

        I_x = fx[i-w:i+w+1, j-w:j+w+1].flatten()
        I_y = fy[i-w:i+w+1, j-w:j+w+1].flatten()
        I_t = ft[i-w:i+w+1, j-w:j+w+1].flatten()

        b = np.reshape(I_t, (I_t.shape[0], 1))
        A = np.vstack((I_x, I_y)).T

        if A.shape[0] > 1:
            U = np.matmul(np.linalg.pinv(A), b)
            u[i, j] = U[0][0]
            v[i, j] = U[1][0]

    return feature_list, u, v


def draw_flow(img, features, u, v, scale=5):
    img_color = cv2.cvtColor(img, cv2.COLOR_GRAY2BGR)
    for pt in features:
        x, y = pt.ravel()
        x, y = int(x), int(y)
        dx = int(u[y, x] * scale)
        dy = int(v[y, x] * scale)
        cv2.arrowedLine(img_color, (x, y), (x+dx, y+dy), (0, 255, 0), 1, tipLength=0.3)
    return img_color


def process_images(image1_path, image2_path):
    old_frame = cv2.imread(image1_path, cv2.IMREAD_GRAYSCALE)
    new_frame = cv2.imread(image2_path, cv2.IMREAD_GRAYSCALE)

    features, u, v = optical_flow(old_frame, new_frame)
    flow_img = draw_flow(new_frame, features, u, v)

    plt.imshow(cv2.cvtColor(flow_img, cv2.COLOR_BGR2RGB))
    plt.title("Optical Flow")
    plt.axis("off")
    plt.show()


def process_video(video_path):
    cap = cv2.VideoCapture(video_path)
    ret, old_frame = cap.read()

    if not ret:
        print("Cannot read video.")
        return

    old_gray = cv2.cvtColor(old_frame, cv2.COLOR_BGR2GRAY)

    while True:
        ret, frame = cap.read()
        if not ret:
            break

        new_gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)
        features, u, v = optical_flow(old_gray, new_gray)
        flow_img = draw_flow(new_gray, features, u, v)

        cv2.imshow("Optical Flow", flow_img)
        if cv2.waitKey(30) & 0xFF == 27:  
            break
        old_gray = new_gray

    cap.release()
    cv2.destroyAllWindows()

process_video("track_Car.avi")

process_images("grove1.png", "grove2.png")

video_path="Cat.mp4" 
video = cv2.VideoCapture(video_path)
ret,frame = video.read()
x_min,y_min,x_max,y_max=36000,36000,0,0


def coordinat_chooser(event,x,y,flags,param):
    global go , x_min , y_min, x_max , y_max
    if event==cv2.EVENT_RBUTTONDOWN:
        x_min=min(x,x_min) 
        y_min=min(y,y_min)
        x_max=max(x,x_max)
        y_max=max(y,y_max)

        cv2.rectangle(frame,(x_min,y_min),(x_max,y_max),(0,255,0),1)
    if event==cv2.EVENT_MBUTTONDOWN:
        print("reset coordinate  data")
        x_min,y_min,x_max,y_max=36000,36000,0,0

cv2.namedWindow('coordinate_screen')
cv2.setMouseCallback('coordinate_screen',coordinat_chooser)


while True:
    cv2.imshow("coordinate_screen",frame)  
    
    k = cv2.waitKey(5) & 0xFF    
    if k == 27:
        cv2.destroyAllWindows()
        break


cv2.destroyAllWindows()
roi_image=frame[y_min:y_max,x_min:x_max]

roi_gray=cv2.cvtColor(roi_image,cv2.COLOR_BGR2GRAY) 

feature_params = dict(maxCorners=20,  
                      qualityLevel=0.2,   
                      minDistance=7,  
                      blockSize=7)

first_gray = cv2.cvtColor(frame,cv2.COLOR_BGR2GRAY)

points = cv2.goodFeaturesToTrack(first_gray, mask=None, **feature_params)

for point in points:
    x, y = point.ravel()
    if y_min <= y <= y_max and x_min <= x <= x_max:
        selected_point = point
        break

if selected_point is not None:
    p0 = np.array([selected_point], dtype=np.float32)

plt.imshow(roi_gray,cmap="gray")

<matplotlib.image.AxesImage at 0x1f73521bc50>

import time

lk_params = dict(winSize=(7, 7),  
                 maxLevel=2,  
                 criteria=(cv2.TERM_CRITERIA_EPS | cv2.TERM_CRITERIA_COUNT, 10, 0.03))

cap = cv2.VideoCapture(video_path)

ret, old_frame = cap.read()

width = old_frame.shape[1]
height = old_frame.shape[0]

mask = np.zeros_like(old_frame)

frame_count = 0
start_time = time.time()

old_gray = first_gray

while True:
    ret, frame = cap.read()
    if not ret:
        break

    frame_gray = cv2.cvtColor(frame, cv2.COLOR_BGR2GRAY)

    if p0 is not None:
        p1, st, err = cv2.calcOpticalFlowPyrLK(old_gray, frame_gray, p0, None, **lk_params)  
        good_new = p1[st == 1]  
        good_old = p0[st == 1]


        if len(good_new) > 0:
            a, b = good_new[0].ravel()
            c, d = good_old[0].ravel()
 
            mask = cv2.line(mask, (int(a), int(b)), (int(c), int(d)), (0, 255, 0), 2)
            frame = cv2.circle(frame, (int(a), int(b)), 5, (0, 255, 0), -1)

            img = cv2.add(frame, mask)

            elapsed_time = time.time() - start_time
            fps = frame_count / elapsed_time if elapsed_time > 0 else 0
            cv2.putText(img, f"FPS: {fps:.2f}", (width - 200, 30), cv2.FONT_HERSHEY_SIMPLEX, 1, (255, 0, 0), 2, cv2.LINE_AA)

            cv2.imshow('frame', img)

            old_gray = frame_gray.copy()
            p0 = good_new.reshape(-1, 1, 2)

        else:
            p0 = None

        if not (25 <= a < width):
            p0 = None

selected_point_distance = 0

    if p0 is None:
        p0 = cv2.goodFeaturesToTrack(frame_gray, mask=None, **feature_params)
        mask = np.zeros_like(frame)
        selected_point_distance=0
 
    frame_count += 1

    k = cv2.waitKey(25)
    if k == 27:
        break

cv2.destroyAllWindows()
cap.release()

Inference

The implemented system successfully tracks a manually selected object, such as a ball, across video frames using the Lucas-Kanade Optical Flow method. By detecting and following strong feature points within a user-defined region of interest (ROI), the algorithm computes the object’s motion and visually represents its trajectory in real time. It also incorporates a recovery mechanism to re-detect features if tracking fails, making it robust for short occlusions or rapid movements. Overall, the approach demonstrates an effective method for motion tracking in videos using classical computer vision techniques.

Result

The estimation of Optical Flow of first two Frames of the video and the full video and Feature Tracking using Kanade Lucas Tomasi Algorithm are implemented successfully.